Symposium: Nanoscience + Engineering Applications

Conference: Active Photonic Platforms (APP) 2022

https://spie.org/OP22N/conferencedetails/active-photonic materials?enableBackToBrowse=true

Titles

- Sub-wavelength custom reprogrammable active photonic platform for high-resolution beam shaping and holography.

Author

# Guillaume Croes,1,2 Robert Gehlhaar,1 and Jan Genoe1,2

# 1 Imec, Kapeldreef 75, 3001, Leuven, Belgium

2 Electrical Engineering Dept. (ESAT), Katholieke Universiteit Leuven, 3001 Leuven, Belgium

Summary (100 words)

Abstract text for online and/or printed programs
Enter 100-word abstract suitable for early release. If your paper is accepted, this 100-word abstract text will be published prior to the meeting in the online (SPIE.org) version of the conference program. A link to it will appear below your paper title and author listing.

We provide insight into the operation and driving mechanism of an electro-optic spatial light modulator based on a Barium Titanate waveguide and an optically transparent electrode cladding layer. We employ electric field simulations, non-linear optics and techniques found in signal analysis to achieve sub-wavelength refractive index modulation of the waveguide. Our generic approach is applicable to Pockels and Kerr cells as well as liquid crystals and is capable of high-quality beam shaping and sensing. In this presentation, we focus on the operation of a reprogrammable Active Photonic Platform in the context of high-quality video holography.

Abstract (250 words)

Abstract text for technical review purposes
Enter 250-word abstract for technical review by the Chairs/Committee. Abstracts should contain enough detail to clearly convey the approach and the results of the research.

High-quality holograms constructed from a modulated waveguide require a periodicity beyond a quarter of the wavelength. Classical electro-optic modulation would be at first glance excluded to achieve this goal, as this requires (metal) electrodes closer than a few tens of nm to the waveguide. However, we enable this in our Active Photonic Platform (APP) by using a metamaterial constructed from a conductive oxide and an isolating nitride to obtain matching refractive index in the waveguide cladding region over the visible wavelength range.

State-of-the-art waveguide-based holographic displays achieve ultra-low noise by using mode conversion as active driving mechanism. Recent work aims to combine this with the precise control offered by subwavelength modulation to create a new generation of APP’s. Our approach employs the electro-optic effect to modulate a slab waveguide by an electrode cladding layer. Crucially, this method enables the use of subwavelength sized electrodes leading to similar sized refractive index modulation, that furthermore is reprogrammable at video rate.

We propose a Barium Titanate waveguide electro-optically modulated on sub-wavelength scale by a metamaterial electrode. We present a driving scheme, inspired by signal analysis, that leverages electric field simulations and nonlinear optics to achieve any desired spatial modulation. We employ finite difference time domain simulations to create dynamic image generation under ideal and realistic scenarios. We considered all relevant design parameters enable the creation of a reprogrammable APP capable of high-quality beamforming, directional sensing, and video-rate holography.